SECTION 10 71 13 - WINDOW SHADES

Draper®, Inc. FlexShade® ZIP XL with Solar Panel

Draper, Inc.; window shade products.

This section is based on the products of Draper, Inc., which is located at:

411 S. Pearl Street

P. O. Box 425

Spiceland, IN 47385-0425

Tel: (800) 238-7999

Fax: (765)987-7142

Email: drapercontract@draperinc.com

Web: www.draperinc.com

Draper is a leader in the manufacture of window shades, projection screens, and gymnasium equipment. Founded in 1902, Draper maintains a tradition of quality and service. Draper products include FlexShade roller-operated and fixed window shades.

Window shades reflect light and heat from the sun, reducing the amount of energy required to cool the building. Motorized window shades can automatically adjust to compensate for the changing shade requirements throughout the day. Solar control shading systems can contribute to LEED points and credits. In addition, shades can reduce or eliminate uncomfortable glare for workers, while still allowing ambient daylighting and view-through to the outside. The construction of window shades has become more friendly to the environment as well by including PVC-free fabrics, recyclable fabrics and lean manufacturing processes.

Our spectrum of fabric colors and styles is unmatched anywhere. And we offer the broadest variety of operator styles in the industry.

This section can be used for specifying the motorized Draper FlexShade ZIP XL with Solar Panel. The system is used as an exterior screen shade system that is very wind resistant.

GENERAL

SECTION INCLUDES

Motorized Exterior Window Shade Zip XL with Solar Panel: Motorized, roll-up fabric window shade system including head box, side channels, motorized operator, solar panel, controls, and mounting hardware.

RELATED SECTIONS

Delete any sections below not relevant to this project; add others as required.

Section 061000 “Rough Carpentry:” Blocking for support of window shade hardware.

Section 079200 “Joint Sealants:” Sealants for perimeter of shade system.

Section 084413 “Glazed Aluminum Curtain Walls:” Openings to receive window shades.

Section 084423 “Structural-Sealant-Glazed Curtain Walls:” Openings to receive window shades.

Section 084113 “Aluminum-Framed Entrances and Storefronts:” Openings to receive window shades.

Section 084126 “All-Glass Entrances and Storefronts:” Openings to receive window shades.

Section 085000 “Windows:” Openings to receive window shades.

Section 085113 “Aluminum Windows:” Openings to receive window shades.

Section 085123 “Steel Windows:” Openings to receive window shades.

Section 085200 “Wood Windows:” Openings to receive window shades.

Section 085313 “Vinyl Windows:” Openings to receive window shades.

Section 085413 “Fiberglass Windows:” Openings to receive window shades.

REFERENCES

Delete references from the list below that are not actually required by the text of the edited section.

NFPA 70 - National Electrical Code.

NFPA 701 - Fire Tests for Flame-Resistant Textiles and Films.

GREENGUARD Gold.

US Green Building Council (USGBC).

SUBMITTALS

Submit under provisions of Section 013300 “Submittal Procedures:”

Product Data: Manufacturer's data sheets on each product specified, including:

Preparation instructions and recommendations.

Installation and maintenance instructions.

Styles, material descriptions, dimensions of individual components, profiles, features, finishes, and operating instructions.

Storage and handling requirements and recommendations.

Mounting details and installation methods.

Typical wiring diagrams including integration of motor controllers with building management system, audiovisual, and lighting control systems as applicable.

Include the following paragraph for projects with complex requirements for window treatment. Delete if not required.

Shop Drawings: Plans, elevations, sections, product details, installation details, operational clearances, wiring diagrams, and relationship to adjacent work.

Delete the following paragraphs if LEED is not applicable.

LEED Submittals: Provide documentation of how the requirements of Credit will be met.

Window Treatment Schedule: For all roller shades. Use same room designations as indicated on the Drawings, field verified window dimensions, quantities, type of shade, controls, fabric, and color, and include opening sizes and key to typical mounting details.

Delete Selection Samples paragraph if colors and shade fabrics have already been selected.

Selection Samples: For each finish product specified, two complete sets of shade cloth options and aluminum finish color samples representing manufacturer's full range of available colors and patterns.

Verification Samples: For each finished product specified, submit two complete sets of shade components, unassembled, demonstrating compliance with specified requirements. Shade fabric sample and aluminum finish sample as selected, representing actual product, color, and patterns. Mark face of material to indicate interior faces.

Maintenance Data: Methods for maintaining motorized exterior window Zip shades, precautions regarding cleaning materials and methods, instructions for operating hardware and controls.

Manufacturer's Certificates: Certify products meet or exceed specified requirements.

QUALITY ASSURANCE

Manufacturer Qualifications: Obtain motorized exterior window zip shades through one source from a single manufacturer with experience in manufacturing products comparable to those specified in this section.

NFPA Flame-Test: Passes NFPA 701. Materials tested shall be identical to products proposed for use.

Shading system shall be UL listed. Provide documentation and proper labeling.

Include a mock-up if the project size and/or quality warrant taking such a precaution. The following is one example of how a mock-up on a large project might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

Mock-Up: Provide a mock-up of one of each type of motorized exterior window zip shade assembly specified for evaluation of mounting, appearance and accessories.

Locate mock-up in [window] [windows] designated by Architect.

Do not proceed with remaining work until mock-up is accepted by Architect.

DELIVERY, STORAGE, AND HANDLING

Do not deliver exterior motorized window zip shades until building is enclosed and construction within spaces where shades will be installed is substantially complete.

Deliver products in manufacturer's original, unopened, undamaged containers with labels intact.

Label containers and shades according to Window Shade Schedule.

Store products in manufacturer's unopened packaging until ready for installation.

SEQUENCING

Ensure that locating templates and other information required for installation of products of this section are furnished to affected trades in time to prevent interruption of construction progress.

Ensure that products of this section are supplied to affected trades in time to prevent interruption of construction progress.

PROJECT CONDITIONS

Install exterior motorized window zip shades after finish work to windows is complete.

WARRANTY

Hardware and Shade Fabric: 10 years from date of Substantial Completion.

Motors and Controls: 5 years from date of Substantial Completion.

PRODUCTS

MANUFACTURERS

Acceptable Manufacturer: Draper®, Inc., which is located at: 411 S. Pearl St.; Spiceland, IN 47385-0425; Toll Free Tel: 800-238-7999; Tel: 765-987-7999; Email: contract@draperinc.com ; Web: www.draperinc.com; Solar Control Solutions.

Requests for substitutions will be considered in accordance with provisions of Section 012500 "Substitution Procedures.”

UL LISTED, EXTERIOR LARGE MOTORIZED FLEXSHADE® ZIP SHADE SYSTEM WITH SOLAR PANEL

Roller Tube: 2.364" (60 mm) dia. Aluminum with 0.049" (1.245 mm) wall; with galvalum coating. Fabric connected to the roller tube with [two-sided tape] [spline groove in which fabric connects].

Headbox: Cassette-style extruded aluminum sections.

Select one or more of the following headbox shapes. If selecting more than one, be sure to indicate clearly on the drawings or describe clearly in the specifications where headboxes of each shape are located.

Size: 5 inches (127 mm) high by 5 inches (127 mm) wide by length required for shade being provided.- Angled Headbox 5 inches (127 mm).

Size: 5 (127 mm) high by 5 (127 mm) wide by length required for shade being provided.- Curved Headbox 5 inches (127 mm).

Side Channels: 1-3/4 inches (45 mm) wide integrated side guide channels. Double chamber, 2-piece fabricated from 0.06 inch (1.5 mm) thick extruded aluminum sections. One chamber accepts zipper retainer in which the zipper edge rides. Other chamber allows for mounting.

Finish: [Textured Slate RAL 7043] [Textured Black RAL 9005] [Textured Sand RAL 1019] [Textured White RAL 9003] [Textured Ivory RAL 1013] [Textured Bronze RAL 7022] [Custom RAL] powder coat finish to meet AAMA 2604 standard for organic coatings.

Zipper retainer: Gray plastic with funnel at top for fabric guiding, cushions are placed in retainer to reduce movement in channel and adjust for fabric shrink.

Hem bar: Extruded aluminum hem bar attached to bottom of shade. Hem bar does not retract into headbox. Hem bar will incorporate steel weights to assist deployment of the system and to tension the fabric when the system is deployed. Plastic end caps will fit flush with side channel. Extra weight in bar is designed to help pull zipper through track.

Channel covers: Injected molded nylon insert to cover bottom of side channels.

Zipper: Zipper is to be heat welded to the edges of fabric. The zipper prevents the fabric from being pulled out of the side channel, and makes the product very wind resistant.

Idle End Assembly: Plastic idle end insert into roller will fit over steel pin and rotate on pin with use of sealed bearing. Steel pin is attached to plastic bracket mounted inside the headbox. Systems not using sealed bearings are not acceptable.

Cradle System: Extrusion supports entire roller using cradle system to reduce roller deflection.

Shade Motor.

Standard Motor: Battery powered Radio Technology motor with built-in radio receiver. 3-wire pigtail allows for attachment of optional three prong plug for plug-an-play operation. Can be tied into building systems using optional external control interfaces. Only those controls listed will operate this motor .

          - Radio Technology Battery Motor- 18 VAC, 50 nM Gaposa Autonomo motor with built-in radio receiver. Tubular motor concealed inside each shade roller tube.

        - Solar Charging Panel with Built in Rechargeable Battery

          - Maximum power current: 305 mA

          - Maximum power voltage: 22 V

          - Maximum power: 6.5 W

          - Protection: IP55

          - Battery Voltage: 16.8V / Ni-MH

          - Capacity: 4200 mAh

          - Working Temperature: -4 degrees F / +158 degrees F

        - Remotes, Switches, and Accessories

Select Remotes from the following list. Delete any not needed.

          - Gaposa Remote – Single Channel

          - Gaposa Remote – 5 Channel

          - Gaposa Remote – 16 Channel

          - Gaposa Remote – Single Channel with Timer

          - Gaposa Wall Mount Wireless Switch– Single Channel

          - Gaposa Wall Mount Wireless Switch – 5 Channel

          - Gaposa Electrical Cord Extension – 18V

          - Bond Bridge Pro

          - Bond Sidekick 5 Channel Wireless Switch

FABRIC

Light-Filtering Fabrics

GreenScreen Revive® 1%. GreenScreen Revive is a fire retardant PVC-free fabric constructed of 100 percent polyester yarn, with a minimum of 89 percent Repreve polyester. Repreve is recycled and recyclable, contains low VOC's, and made in the USA. Fire rating: NFPA 701-99 TM#1, California U.S. Title 19, Cradle to Cradle Certified Silver, GREENGUARD®, GREENGUARD Gold®. Approximate Openness Factor: 1 percent. Average Fabric Thickness: 0.019 inch (0.48 mm). Average Fabric Weight: 5.87 ounces per square yard.

GreenScreen Revive® 5%. GreenScreen Revive is a fire retardant PVC-free fabric is constructed of 100 percent polyester yarn, with a minimum of 89 percent Repreve polyester. Repreve is recycled and recyclable, contains low VOC's, and made in the USA. Fire rating: NFPA 701-99 TM#1, California U.S. Title 19, Cradle to Cradle Certified Silver, GREENGUARD, GREENGUARD Gold, Green PVC Free. Approximate Openness Factor: 5 percent. Average Fabric Thickness: 0.017 inch (.043 mm). Average Fabric Weight: 5.01 ounces per square yard.

GreenScreen Evolve® 3%: GreenScreen Evolve is a fire retardant PVC-free fabric is constructed of 100 percent polyester yarn, with up to 78% Repreve polyester. Repreve is recycled and recyclable, contains low VOC's, and made in the USA. Fire rating: NFPA 701-99 TM#1, California U.S. Title 19, Cradle to Cradle Certified Silver, GREENGUARD®, GREENGUARD Gold®, PVC Free. Approximate Openness Factor: 3 percent. Average Fabric Thickness: 0.027 inch (.068 mm). Average Fabric Weight: 8.41 ounces per square yard. Microbial Resistance, ASTM E2180 and ASTM G21: Pass.

GreenScreen® Nature™ 5%: GreenScreen Nature is an inherently fire-retardant fabric constructed of 100 percent fiberglass yarn in a mock leno weave. Additional fire retardancy chemicals are not required. Nature is recyclable, contains low VOCs, is halogen- and PVC-free, and is Red List compliant with an HPD available, and made in the USA. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109 Small & Large Scale. GREENGUARD®, GREENGUARD Gold®. Approximate Openness Factor: 5 percent. Average Fabric Thickness: 0.010 inch (.024 mm). Average Fabric Weight: 5.10 ounces per square yard (174 grams per square meter). Microbial Resistance, ASTM E2180 and ASTM G21: Pass.

SheerWeave® Infinity2 sustainable window treatment fabric eco-friendly basketweave. Core yarn and coating are PVC-free, lead-free and 100 percent recycable. Fire classification: ASTM E-84 (Class I), NFPA 701-2004 TM#1 (small scale), NFPA 101 (Class A Rating) and CAN/ULC-S 109-03 Large, GREENGUARD®, GREENGUARD Gold®.

Average 1 percent open. Average Fabric Thickness: .033 inch (.838 mm) Average Fabric Weight: 14.63 ounces per square yard. Available in 63 inch and 98 inch (1600 mm and 2489 mm) roll widths.

Average 3 percent open. Average Fabric Thickness: .031 inch (.79 mm) Average Fabric Weight: 13.69 ounces per square yard. Available in 63 inch, 84 inch and 98 inch (1600 mm, 2134 mm and 2489 mm) roll widths.

Average 5 percent open. Average Fabric Thickness: .03 inch (.76 mm). Average Fabric Weight: 12.82 ounces per square yard. Available in 63 inch, 84 inch and 98 inch (1600 mm, 2134 mm and 2489 mm) roll widths.

SheerWeave® Basic 3% by Phifer®. An economical alternative to traditional solar screen fabrics. Fire Rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), NFPA 701 TM#2 (large scale), CAN/CGSB2-4.162-M80. Bacteria and Fungal Resistance	ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental Certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. Average 3 percent Openness. Average Fabric Thickness: .025 inch. Average Fabric Weight: 16.4 ounces per square yard.

SheerWeave® Basic 5% by Phifer®. An economical alternative to traditional solar screen fabrics. Fire Rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), NFPA 701 TM#2 (large scale), CAN/CGSB2-4.162-M80. Bacteria and Fungal Resistance	ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental Certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. Average 5 percent Openness. Average Fabric Thickness: 0.022 inch. Average Fabric Weight: 14.1 ounces per square yard.

SheerWeave® Series SW2000 by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 by 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 14.26 oz/sq yd, .019 inches thick. Series SW2000 average 5 percent open.

SheerWeave® Series SW2100 by Phifer®: VOC Emissions: GREENGUARD Gold®. Manufacturer to supply GREENGUARD Gold 500 denier fiberglass, vinyl coated and woven into a 2 by 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 14.26 oz/sq yd, .019 inches thick. Series SW2100 average 10 percent open.

SheerWeave® Series SW2390 Performance+ by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave with metalized coating on outward side. Fire rating: California U.S. Title 19 (small scale), NFPA 101 (Class A Rating), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80, NFPA 701 TM#1 (small scale), IBC Section 803.1.1 (Class A Rating), NFPA 701 TM#2 (large scale). Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 5 percent open. .017 inches thick. 11.8 oz/square yard.

SheerWeave® Series SW2400 (2410) by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 3 percent open .019 inches thick. 14.1 oz/square yard.

SheerWeave® Series SW2410 Performance+ by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave with metalized coating on outward side. Fire rating: California U.S. Title 19 (small scale), NFPA 101 (Class A Rating), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80, NFPA 701 TM#1 (small scale), IBC Section 803.1.1 (Class A Rating), NFPA 701 TM#2 (large scale). Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 3 percent open .019 inches thick. 14.1 oz/square yard.

SheerWeave® Series SW2500 by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 1 percent open, .024 inches thick. 16.39 oz/square yard.

SheerWeave® Series SW2500 Performance+ by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave with metalized coating on outward side. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 1 percent open, .024 inches thick. 16.39 oz/square yard.

SheerWeave® Series SW2600 (2360) by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban ®antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 10 percent open, .017 inches thick. 10.4 oz/square yard.

SheerWeave® Series SW2701 by Phifer®: Duplex basketweave fabric—light exterior color combined with dark interior color for thermal comfort and view-through. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental Certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 1 percent open. .027 inches thick. 14.6 oz/square yard.

SheerWeave® Series SW2703 by Phifer®: Duplex basketweave fabric—light exterior color combined with dark interior color for thermal comfort and view-through. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80, CAN/ULC S-102. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. 3 percent open. .028 inches thick. 14.0 oz/square yard.

SheerWeave® Series SW2705 by Phifer®: Duplex basketweave fabric—light exterior color combined with dark interior color for thermal comfort and view-through. GREENGUARD Gold®. Manufacturer to supply GREENGUARD Gold certificate. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental Certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 5 percent open. .025 inches thick. 11.8 oz/square yard.

SheerWeave® Series SW2710 by Phifer®: Duplex basketweave fabric—light exterior color combined with dark interior color for thermal comfort and view-through. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental Certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 10 percent open. .023 inches thick. 10.4 oz/square yard.

SheerWeave® Series SW2900 (2390) by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 101 (Class A Rating), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80, NFPA 701 TM#1 (small scale), IBC Section 803.1.1 (Class A Rating), NFPA 701 TM#2 (large scale). Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 5 percent open. .017 inches thick. 11.8 oz/square yard.

SheerWeave® Series SW3000 by Phifer®: Vinyl coated fiberglass and vinyl coated polyester woven into 62 by 20 mesh. Fire rating: NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), NFPA 701 TM#2 (large scale). Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 13.46 oz/sq yd, .028 inches thick. Average 14 percent open.

SheerWeave® Series PW3500 (4000) by Phifer®: Vinyl coated polyester yarn woven into basketweave pattern. Uses DOW ECOLIBRIUM™ bio-based, phthalate-free plasticizer. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental Certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content, REACH (EC 1907/2006) compliant. Average 5 percent open, 19.2 oz/sq yd, .036 inches thick.

SheerWeave® Series PW4100 by Phifer®: Vinyl coated polyester yarn woven into basketweave pattern. Uses DOW ECOLIBRIUM™ bio-based, phthalate-free plasticizer. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental Certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content, REACH (EC 1907/2006) compliant. Average 10 percent open, 17.5 oz/sq yd, .035 inches thick.

SheerWeave® Series PW4400 by Phifer®: Vinyl coated polyester yarn woven into basketweave pattern. Uses DOW ECOLIBRIUM™ bio-based, phthalate-free plasticizer. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental Certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content, REACH (EC 1907/2006) compliant. Average 3 percent open, 20.7 oz/sq yd, .037 inches thick.

SheerWeave® Series PW4500 by Phifer®: Vinyl coated polyester yarn woven into basketweave pattern. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale). Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. Average 5 percent open, 14.4 oz/sq yd, .024 inches thick.

SheerWeave® Series PW4600 by Phifer®: Vinyl coated polyester yarn woven into basketweave pattern. Fire rating: Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale). Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. Average 3 percent open, 17.4 oz/sq yd, .030 inches thick.

SheerWeave® Series PW4550 by Phifer®: Vinyl coated polyester yarn woven into basketweave pattern. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale). Bacteria and fungal Resistance. ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. Average 5 percent open, 12.5 oz/sq yd, .025 inches thick.

SheerWeave® Series PW4650 by Phifer®: Vinyl coated polyester yarn woven into basketweave pattern. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale). Bacteria and fungal Resistance. ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. Average. 3 percent open, 13.3 oz/sq yd, .026 inches thick.

SheerWeave® PW4800 by Phifer®: Vinyl coated polyester with maximum UV blockage. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 1 percent openness factor. 18.5 oz/sq yd, .036 inches thick.

SheerWeave® PW4901 by Phifer®: Vinyl coated polyester with maximum UV blockage. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. UV protection: Seal of Approval by the Melanoma International Foundation (MIF) for effectiveness in preventing sun damage to the skin or eyes. 1 percent openness factor. 15.85 oz/sq yd, .028 inches thick.

SheerWeave® PW4903 by Phifer®: Vinyl coated polyester with maximum UV blockage. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. UV protection: Seal of Approval by the Melanoma International Foundation (MIF) for effectiveness in preventing sun damage to the skin or eyes. 3 percent openness factor. 14.9 oz/sq yd, .027 inches thick.

SheerWeave® Series SW5000 by Phifer®: Polyester and vinyl coated polyester woven into a 42 by 20 mesh. Fire rating: California Technical Bulletin 117 Sect. E. Part 1/ /UBC (Class 1)/British Standard 5867. Varies depending on color. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, ASTM D3273, AATCC 30 Part 3, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 12.3 to14.5 oz/sq yd, .035 inches - .044 inches thick, depending on color. 5 to 10 percent open, depending on color.

SheerWeave® Series SW7450 by Phifer®: PVC-free polyester with acrylic coating. Fire rating: California Technical Bulletin 117, IBC Section 803.1.1 (Class A Rating), NFPA 101 (Class A Rating). Bacteria and Fungal Resistance: ASTM G21-96, AATCC 174-1998 Part II and III. Environmental Certification: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe Use: RoHS/Directive 2002/95/EC, ANSI/WCMA A 100.1-2007 for lead content, US Consumer Product Safety Commission Section 101. UV Protection: Seal of Approval by the Melanoma International Foundation (MIF) for effectiveness in preventing sun damage to the skin or eyes. .026 inches thick, 6.54 oz/square yard.

SheerWeave® Series SW7650 by Phifer®: PVC-free polyester. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale). Bacteria and fungal resistance: ASTM G21-96, AATCC 174-1998 Part II and III. Environmental certification: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, ANSI/WCMA A 100.1-2007 for lead content, US Consumer Product Safety Commission Section 101. UV protection: Seal of Approval by the Melanoma International Foundation (MIF) for effectiveness in preventing sun damage to the skin or eyes. .014 inches thick, 6.2 oz/square yard.

SheerWeave® Series SW8000 by Phifer®: PVC-free polyester. Fire rating: California U.S. Title 19 (small scale), BS 5867 2008 Part 2 Type B Performance, IBC Section 803.1.1 (Class A Rating), NFPA 701 TM#1 (small scale), NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80, B1 (DIN 4102-1), NFPA 101 (Class A Rating). Bacteria and fungal resistance: ASTM E 2180, ASTM G21. Environmental certification: Cradle to Cradle Certified™ Bronze and certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, ANSI/WCMA A 100.1-2007 for lead content, US Consumer Product Safety Commission Section 101and REACH (EC 1907/2006) compliant. UV Protection: Seal of Approval by the Melanoma International Foundation (MIF) for effectiveness in preventing sun damage to the skin or eyes. Average 3% open. .019 inches thick. 7.35 oz/square yard.

E Screen™ 1% by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 1 percent open, .020 inches thick, 13.3 oz/square yard.

E Screen™ 3% by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 3 percent open, .017 inches thick, 11.6 oz/square yard.

E Screen™ 5% by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 5 percent open, .016 inches thick, 10.7 oz/square yard.

E Screen™ 10% by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. .016 inches thick. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 10 percent open, .020 inches thick, 10.3 oz/square yd.

E Screen™ 1% with KOOLBLACK® by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 1 percent open, .021 inches thick. Weight: 12.95 oz/square yard.

E Screen™ with KOOLBLACK®. 3 percent openness factor by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 3 percent open, .020 inches thick, 11.3 oz. per square yard.

E Screen™ with KOOLBLACK®. 5 percent openness factor by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 5 percent open, .019 inches thick, 10.6 oz/square yard.

E Screen Deco™ 1% by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 1 percent open, .022 inches thick. 12.30 oz/square yard.

E Screen Deco™ 3% by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 3 percent open, .019 inches thick. 11.50 oz/square yard.

M Screen™ 1% by Mermet®: PVC coated fiberglass in 1 by 2 weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 1 percent open, .022 inches thick, 13.12 oz/square yard.

M Screen™ 3% by Mermet®: PVC coated fiberglass in 1 by 2 weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 3 percent open, .022 inches thick, 12.27 oz/square yard.

M Screen™ 5% by Mermet®: PVC coated fiberglass in 1 by 2 weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 5 percent open, .022 inches thick, 11.3 oz/square yard.

M Screen Chroma™ 3% by Mermet®: PVC coated fiberglass in 1 by 2 rib weave with metalized coating on outward side. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 3 percent open, .018 inches thick, 12.0 oz/square yard.

M Screen Deco™ 3% by Mermet®: PVC coated fiberglass in a rib weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 3 percent open, .023 inches thick, 12.721 oz/square yd.

M Screen Deco™ 5% by Mermet®: Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 5 percent open, .020 inches thick, 11.3 oz/square yd.

S Screen™ 1% by Mermet®: PVC coated fiberglass wrapped in polyester thread in a plain weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 3 percent open, .041 inches thick, 19.10 oz/square yard.

S Screen™ 4% by Mermet®: PVC coated fiberglass wrapped in polyester thread in a plain weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. ASTM E2180 and ASTM G21. Environmental benefits: RoHS – lead free. 4 percent open, .037 inches thick, 18.9 oz/square yard.

S Screen Naturals™ 5% by Mermet®: PVC coated fiberglass wrapped in polyester thread in a fancy weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 5 percent open, .042 inches thick 15 oz/square yard.

T Screen™ 3% by Mermet®: PVC coated fiberglass woven mesh. Dark on one side, light on the other. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 3 percent open, .025 inches thick, 11.6 oz/square yard.

T Screen Deco™ 1% by Mermet®: PVC coated fiberglass satin style. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 1 percent open, .028 inches thick, 13.76 oz/square yd.

T Screen Deco™ 3% by Mermet®: PVC coated fiberglass satin style. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 3 percent open, .028 inches thick, 12.98 oz/square yard.

T Screen Deco™ 5% by Mermet®: PVC coated fiberglass satin style. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 5 percent open, .028 inches thick, 11.89 oz/square yard.

T Screen™ with KOOLBLACK® 1% by Mermet®: PVC coated fiberglass woven mesh. Dark on one side, light on the other. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 1 percent open, .024 inches thick, 13.54 oz/square yard.

T Screen™ with KOOLBLACK® 3% by Mermet®: PVC coated fiberglass woven mesh. Dark on one side, light on the other. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 3 percent open, .029 inches thick, 13.27 oz/square yard.

T Screen™ with KOOLBLACK®. 5 percent openness factor by Mermet®: PVC coated fiberglass woven mesh. Dark on one side, light on the other. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 5 percent open, .027 inches thick, 12.48 oz/ square yard.

T Screen Naturals™ 1% by Mermet®: PVC coated fiberglass satin style. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 1 percent open, .027 inches thick, 13.83 oz/square yard.

T Screen Naturals™ 3% by Mermet®: PVC coated fiberglass satin style. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 3 percent open, .027 inches thick, 12.59 oz/ square yard.

T Screen Naturals™ 5% by Mermet®: PVC coated fiberglass satin style. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 5 percent open, .028 inches thick, 12.24 oz/ square yard.

Soltis® Perform 92 4% by Serge Ferrari®: PVC-coated polyester with silver backing on outside and pre-tensioning process for dimensional stability. Fire rating: Method 1/NFPA 701, CSFM T19, Class A/ ASTM E84. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Recyclable through TexyLoop. 4 percent open, .018 inches thick, 12.4 oz/square yard.

Soltis® Master 99 3% by Serge Ferrari®: PVC-coated polyester with silver backing on outside and pre-tensioning process for dimensional stability. Fire rating: Method 1/NFPA 701, CSFM T19, Class A/ ASTM E84. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Recyclable through TexyLoop. 3 percent open, .0156 inches thick, 8.6 oz/ square yard.

SilverScreen 4% by Verosol: 36 percent fiberglass and 64 percent vinyl with an ultra-fine layer of aluminum on the backside. Fire rating: NFP 92503 specification M1, NFPA 701, B5 5867 Part 2 Type B, AS 1530. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant - Lead Free, Formaldehyde free. 5 percent Open, 0.020 inches thick, 11.7974 oz/square yard.

Room Darkening Fabrics

SunBloc Series SB9000: Close woven fiberglass base textile with sun-resistant vinyl film bonded to each side, opaque with minimum tensile strength of 190 pounds for warp and 180 pounds for fill. Fire rating: NFPA 701 1006-Test 1, California U.S. Title 19. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. REACH and RoHS compliant - Lead Free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Washable and stain resistant. Same color both sides. Opaque, .015 inches thick, 12 oz/square yard.

SunBloc Series SB9100: Close woven fiberglass base textile with sun-resistant vinyl film bonded to each side, opaque with minimum tensile strength of 190 pounds for warp and 180 pounds for fill. Fire rating: NFPA 701 1006-Test 1, California U.S. Title 19. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. REACH and RoHS compliant - Lead Free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Washable and stain resistant. White exterior in all colors. Opaque, .013 inches thick, 12 oz/square yard.

Apagón Style III is a 3-ply opaque shade fabric consisting of vinyl, fiberglass, and a proprietary coating for maximum blackout. Thread Count: 59 threads per square inch. Fire rating: NFPA 701-2019 Test Method 1. Environmental Benefits: ASTM D3335 85a - Lead Testing. Bacterial and fungal resistance: ASTM Method G-21-15 – Resistance to Fungi. Opaque, .0131 inches thick, 12.8 oz/square yard.

Flocké® by Mermet®: Opaque, PVC-free fiberglass textile with acrylic flocked backing. Antistatic treated. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Draper shades made with this fabric are GreenSpec® listed. Opaque, .020 inches thick, 15.93 oz/square yard.

SheerWeave® Series SW7000 Blackout by Phifer®: PVC-free polyester with acrylic foamed backing. Fire rating: California U.S. Title 19 (small scale), BS 5867 Part 2 Type B Performance, IBC Section 803.1.1 (Class A Rating), NFPA 101 (Class A Rating), NFPA 701 TM#1 (small scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacteria and fungal resistance: ASTM G21, AATCC 174-1998 Part II and III. Environmental certification: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content. Draper shades made with this fabric are GreenSpec® listed. Opaque, .030 inches thick, 13.92 oz/square yard.

SheerWeave® Series SW7100 Blackout by Phifer®: PVC-coated Fiberglass laminated with a 2-ply PVC film. Fire rating: NFPA 701 TM#1 (small scale), NFPA 701 TM#2 (large scale), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), NFPA 101 (Class B Rating), IBC Section 803.1.1 (Class B Rating). Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; Style 2000 face includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe Use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. Opaque, .025 inches thick, 21.6 oz/square yard, opaque.

SheerWeave® Series SW7400 by Phifer®: PVC-free polyester with acrylic foamed backing. Fire rating: California U.S. Title 19 (small scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB 2-4. 162-M80, NFPA 701 TM#1 (small scale), NFPA 701 TM#2 (large scale). Bacteria and Fungal Resistance: ASTM G21-96, AATCC 174-1998 Part II and III. Environmental Certification: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe Use: RoHS/Directive 2002/95/EC, ANSI/WCMA A 100.1-2007 for lead content, US Consumer Product Safety Commission Section 101. UV Protection: Seal of Approval by the Melanoma International Foundation (MIF) for effectiveness in preventing sun damage to the skin or eyes. Opaque, .029 inches thick, 12.53 oz/square yard.

SheerWeave® Series SW7500 by Phifer®: PVC-free polyester with an acrylic coating. Fire rating: California U.S. Title 19 (small scale), NFPA 101 (Class A Rating), NFPA 701 TM#1 (small scale), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), IBC Section 803.1.1 (Class A Rating), CAN/CGSB 2-4.162-M80. Bacterial and fungal resistance: ASTM E 2180 and ASTM G21. Environmental certification: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content. Opaque, .016 inches thick, 12.14 oz/square yard.

SheerWeave® Series SW7600 by Phifer®: PVC-free polyester with an acrylic foamed backing. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale). Bacteria and fungal resistance: ASTM G21-96, AATCC 174-1998 Part II and III. Environmental certification: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, ANSI/WCMA A 100.1-2007 for lead content, US Consumer Product Safety Commission Section 101. UV protection: Seal of approval by the Melanoma International Foundation (MIF) for effectiveness in preventing sun damage to the skin or eyes. Opaque, .020 inches thick, 11.8 oz/square yard.

Avila Twilight™ by Mermet®. 37.5 percent Polyester and 62.5 percent acrylic with foam backing. Duraguard fabric protector and Sanitized Antimicrobial Protection. Plain weave that is 100% PVC free. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 0% open, .020 inches thick, 14.5 oz/square yard.

Vizela™ < 1% by Mermet®: 100 percent fiberglass with an EVA coating and PVC free. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. < 1% open, .013 inches thick, 8.7 oz/square yard.

Soltis® Opaque B92 by Serge Ferrari®. PVC-coated polyester with silver backing on outside and pre-tensioning process for dimensional stability. Fire rating: Method 1 and 2/NFPA 701, Class A / ASTM E84. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Recyclable through TexyLoop. 0 percent open, .0312 inches thick, 19.1 oz/square yard.

Retain the Window Size paragraph below if window size is known and uniform throughout the project. If there are different window sizes in the project, copy and paste this paragraph once for each different size; designate a different window opening type for each different size; and provide the dimension for each window type.

If window sizes are unknown or there is a large number of different size windows, delete the paragraph below and be sure the window sizes are clearly indicated on the Drawings.

Window <Insert window designation used on Drawings> Size: <insert dimension> inches wide by <insert dimension> inches tall.

Custom Printed Graphics: As submitted to manufacturer in accordance with manufacturer’s guidelines.

EXECUTION

EXAMINATION

Do not begin installation until substrates have been properly prepared.

If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

PREPARATION

Coordinate requirements for blocking and structural supports to ensure adequate means for installation of window shades.

Coordinate requirements for power supply conduit, and wiring required for exterior motorized window Zip shades and controls.

INSTALLATION

Install in accordance with manufacturer's instructions.

Install exterior motorized window Zip shades level, plumb, square, and true. Allow proper clearances for window operation hardware.

Position shades level, plumb, and at proper height relative to adjacent construction. Secure with fasteners recommended by manufacturer.

Include the following paragraph for if systems are to be used for high levels of light exclusion. Delete if not applicable.

Install headbox, side channels, and sill channel with sealant specified in Section 079200 “Joint Sealants” to eliminate light leaks at perimeter of shade system.

TESTING AND DEMONSTRATION

Test exterior motorized window Zip shades to verify that operating mechanism, fabric retainer, and other operating components are functional. Correct deficiencies.

Demonstrate operation of shades to Owner's designated representatives.

PROTECTION

Protect installed products until completion of project.

Touch-up, repair or replace damaged products before Substantial Completion.

SCHEDULES

Retain Paragraph below to suit project requirements. Identify products by name on the Drawings or alternately use this paragraph to define the location of each type of material to be used. Delete if not required.

Refer to Drawings for exterior motorized window Zip shade types and locations.

END OF SECTION