SECTION 12 24 00 - WINDOW SHADES

Draper®, Inc. FlexShade® Tension ZIP

Draper®, Inc.; window shade products.

This section is based on the products of Draper, Inc., which is located at:

411 S. Pearl Street

P. O. Box 425

Spiceland, IN 47385-0425

Tel: (800) 238-7999

Fax: (765)987-7142

Email: drapercontract@draperinc.com

Web: www.draperinc.com

Draper is a leader in the manufacture of window shades, projection screens, and gymnasium equipment. Founded in 1902, Draper maintains a tradition of quality and service. Draper products include FlexShade roller-operated and fixed window shades.

Window shades reflect light and heat from the sun, reducing the amount of energy required to cool the building. Motorized window shades can automatically adjust to compensate for the changing shade requirements throughout the day. Solar control shading systems can contribute to LEED points and credits. In addition, shades can reduce or eliminate uncomfortable glare for workers, while still allowing ambient sunlighting and view-through to the outside. The construction of window shades has become more friendly to the environment as well by including PVC-free fabrics, recyclable fabrics and lean manufacturing processes.

Our spectrum of fabric colors and styles is unmatched anywhere. And we offer the broadest variety of operator styles in the industry.

This section can be used for specifying the motorized Draper FlexShade Tension ZIP system.

GENERAL

SECTION INCLUDES

Motorized Tension ZIP Shade System: Electrically operated, motorized, horizontal roll-up, fabric shading system. Assembly includes motor operator, controls, mounting hardware, and other components necessary for complete installation.

RELATED SECTIONS

Delete any sections below not relevant to this project; add others as required.

Section 061000 “Rough Carpentry:” Blocking for support of window shade hardware.

Section 079200 “Joint Sealants:” Sealants for perimeter of shade system.

Section 084413 “Glazed Aluminum Curtain Walls:” Openings to receive window shades.

Section 084423 “Structural-Sealant-Glazed Curtain Walls:” Openings to receive window shades.

Section 084113 “Aluminum-Framed Entrances and Storefronts:” Openings to receive window shades.

Section 084126 “All-Glass Entrances and Storefronts:” Openings to receive window shades.

Section 085000 “Windows:” Openings to receive window shades.

Section 085113 “Aluminum Windows:” Openings to receive window shades.

Section 085123 “Steel Windows:” Openings to receive window shades.

Section 085200 “Wood Windows:” Openings to receive window shades.

Section 085313 “Vinyl Windows:” Openings to receive window shades.

Section 085413 “Fiberglass Windows:” Openings to receive window shades.

Division 26 - Electrical: Electrical supply, conduit, and wiring for motorized window shades.

REFERENCES

Delete references from the list below that are not actually required by the text of the edited section.

NFPA 70 - National Electrical Code.

NFPA 701 - Fire Tests for Flame-Resistant Textiles and Films.

[GREENGUARD] [GREENGUARD Gold].

US Green Building Council (USGBC).

SUBMITTALS

Submit under provisions of Section 013300 “Submittal Procedures:”

Product Data: Manufacturer's data sheets on each product specified, including:

Preparation instructions and recommendations.

Installation and maintenance instructions.

Styles, material descriptions, dimensions of individual components, profiles, features, finishes, and operating instructions.

Storage and handling requirements and recommendations.

Mounting details and installation methods.

Typical wiring diagrams including integration of motor controllers with building management system, audiovisual, and lighting control systems as applicable.

Include the following paragraph for projects with complex requirements for window treatment. Delete if not required.

Shop Drawings: Plans, elevations, sections, product details, installation details, operational clearances, wiring diagrams, and relationship to adjacent work.

Delete the following paragraphs if LEED is not applicable.

LEED Submittals: Provide documentation of how the requirements of Credit will be met.

Window Treatment Schedule: For all roller shades. Use same room designations as indicated on the Drawings, field verified window dimensions, quantities, type of shade, controls, fabric, and color, and include opening sizes and key to typical mounting details.

Delete Selection Samples paragraph if colors and shade fabrics have already been selected.

Selection Samples: For each finish product specified, two complete sets of shade cloth options and aluminum finish color samples representing manufacturer's full range of available colors and patterns.

Verification Samples: For each finished product specified, submit two complete sets of shade components, unassembled, demonstrating compliance with specified requirements. Shade fabric sample and aluminum finish sample as selected, representing actual product, color, and patterns. Mark face of material to indicate interior faces.

Maintenance Data: Methods for maintaining motorized exterior window Zip shades, precautions regarding cleaning materials and methods, instructions for operating hardware and controls.

Manufacturer's Certificates: Certify products meet or exceed specified requirements.

QUALITY ASSURANCE

Manufacturer Qualifications: Obtain motorized exterior window zip shades through one source from a single manufacturer with experience in manufacturing products comparable to those specified in this section.

NFPA Flame-Test: Passes NFPA 701. Materials tested shall be identical to products proposed for use.

Wind Tunnel Test: Supply wind tunnel test results from an independent, professional wind testing facility.

Shading system shall be UL listed. Provide documentation and proper labeling.

Include a mock-up if the project size and/or quality warrant taking such a precaution. The following is one example of how a mock-up on a large project might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

Mock-Up: Provide a mock-up of one of each type of motorized exterior window Zip shade assembly specified for evaluation of mounting, appearance and accessories.

Locate mock-up in [window] [windows] designated by Architect.

Do not proceed with remaining work until mock-up is accepted by Architect.

DELIVERY, STORAGE, AND HANDLING

Do not deliver exterior motorized window zip shades until building is enclosed and construction within spaces where shades will be installed is substantially complete.

Deliver products in manufacturer's original, unopened, undamaged containers with labels intact.

Label containers and shades according to Window Shade Schedule.

Store products in manufacturer's unopened packaging until ready for installation.

SEQUENCING

Ensure that locating templates and other information required for installation of products of this section are furnished to affected trades in time to prevent interruption of construction progress.

Ensure that products of this section are supplied to affected trades in time to prevent interruption of construction progress.

PROJECT CONDITIONS

Install exterior motorized window zip shades after finish work to windows is complete.

WARRANTY

Hardware and Shade Fabric: Draper standard one-year limited warranty.

Motors and Controls: Draper standard five-year limited warranty.

PRODUCTS

MANUFACTURERS

Acceptable Manufacturer: Draper®, Inc., which is located at: 411 S. Pearl St., Spiceland, IN 47385-0425; Toll Free Tel: 800-238-7999; Tel: 765-987-7999; Email: contract@draperinc.com; Web: www.draperinc.com; Solar Control Solutions.

Requests for substitutions will be considered in accordance with provisions of Section 012500 "Substitution Procedures.”

UL LISTED EXTERIOR MOTORIZED FLEXSHADE® TENSION ZIP SHADE

Roller Tube: Fabricated from aluminum, 3-inch (76 mm) (at manufacturer’s discretion) outside diameter to accommodate shade type and size. Roller assembly is integrated with floating bracketry. Fabric is connected to the roller tube with spline groove in which fabric connects.

Headbox: Consists of cassette-style extruded aluminum sections with endcaps.

Select one or more of the following headbox sizes and shapes. If selecting more than one, be sure to indicate clearly on the drawings or describe clearly in the specifications where headboxes of each size and shape are located.

Size: 5-1/4 inches (133 mm) high by 5-1/4 inches (133 mm) wide by length required for shade being provided. Curved Headbox.

Size: 5-1/4 inches (133 mm) high by 5-1/4 inches (133 mm) wide by length required for shade being provided. Angled Headbox.

Side Channels: 2-7/8 inches (73 mm) wide by 1-3/4 inches (44 mm) integrated side guide channels. Extruded aluminum side channels incorporate plastic inner channels. Plastic inner channels held inside the extrusion by means of cushioning pads that also dampen the movement of the fabric panel under wind load. ZIP detail on the edge of the fabric panel runs inside inner channel to retain the fabric in place.

Zipper retainer: Grey plastic with funnel at top for fabric guiding, spring clips are placed in retainer to reduce movement in channel and adjust for fabric shrink.

Zipper: Zipper is to be heat welded to the edges of fabric. The zipper prevents the fabric from being pulled out of the side channel and makes the product very wind resistant.

Hem bar: Weighted extruded aluminum 2 inches (51 mm) x 4 inches (105 mm) hem bar attached to bottom of shade. Hem bar does not retract into headbox. Hem bar to include tensioning springs, pulleys and tie-off cleats used to set cable tension.

Tensioning Cable: Cable to be Grey, 3 mm diameter, lightweight and high strength. 12-strand single braid Dyneema® SK78, with tough ArmourCoat® finish.

Mounting Brackets: Mounting Brackets are constructed of steel.

Spreader Bar: Spreader bar is constructed of 2-inch (51mm) diameter aluminum tube. Used when shade fabric tension distorts the side channels, interfering with shade operation.

Channel endcaps: Injection molded nylon insert to cover bottom of side channels.

Finish: [Textured Slate RAL 7043] [Textured Black RAL 9005] [Textured Sand RAL 1019] [Textured White RAL 9003] [Textured Ivory RAL 1013] [Textured Bronze RAL 7022] [Custom RAL] powder coat finish to meet AAMA 2604 standard for organic coatings.

Idle End Assembly: Plastic idler end insert fits over steel pin and rotates on the pin with use of sealed bearing. Steel pin is attached to plastic bracket mounted inside the headbox. Systems not using sealed bearings are not acceptable.

Shade Motor.

Standard Motor: Radio Technology Motor with built-in radio receiver. 3-wire pigtail allows for attachment of optional three prong plug for plug-and-play operation. Can be tied into building systems using optional external control interfaces.

Standard Motor: Radio Technology Motor- 110 VAC motor with built-in radio receiver. Tubular motor concealed inside each shade roller tube. Lower RPM motor required.

Optional Motor: 110V AC tubular motor with 4-wire pigtail. All controls and receivers will be external to the motor. Can be tied into building systems using optional external control interfaces.

          - Optional Motor: 110 VAC, single phase, 60 HZ, instantly reversible, lifetime lubricated, and equipped with internal thermal overload protector, electric brake, and pre-set accessible limit switches. Tubular motor concealed inside each shade roller tube.

        - RTS Controls

Select controls from the following list. Delete any not needed.

          - Decora RTS 1-5 Channel Wall Switch Kit

          - Situo 1 Single Channel Handheld Remote (indoor only)

          - Telis 1 Single Channel Handheld Remote (indoor only)

          - Situo 5 Five Channel Handheld Remote (indoor only)

          - Telis 4 Five Channel Handheld Remote (indoor only)

          - Telis 16 Sixteen Channel Remote (indoor only)

          - Telis 1 Patio Remote

          - Telis 4 Patio Remote

          - Eolis RTS Wind Sensor

          - Soliris RTS Wind/Sun Sensor

          - Situo 5 Soliris Remote

          - TaHoma (indoor only)

FABRIC

Light-Filtering Fabrics

SheerWeave® Series SW2000 by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 by 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacterial and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacterial Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 14.26 oz/sq yd, .019 inches thick. Series SW2000 average 5 percent open.

SheerWeave® Series SW2100 by Phifer®: VOC Emissions: GREENGUARD Gold®. Manufacturer to supply GREENGUARD Gold 500 denier fiberglass, vinyl coated and woven into a 2 by 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacterial and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacterial Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 14.26 oz/sq yd, .019 inches thick. Series SW2100 average 10 percent open.

SheerWeave® Series SW2400 (2410) by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacterial and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacterial Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 3 percent open .019 inches thick. 14.1 oz/square yard.

SheerWeave® Series SW2500 by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacterial and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacterial Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 1 percent open, .024 inches thick. 16.39 oz/square yard.

SheerWeave® Series SW2600 (2360) by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacterial and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacterial Standard ASTM 6329; includes Microban ®antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 10 percent open, .017 inches thick. 10.4 oz/square yard.

SheerWeave® Series SW2701 by Phifer®: Duplex basketweave fabric—light exterior color combined with dark interior color for thermal comfort and view-through. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacterial and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacterial Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental Certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 1 percent open. .027 inches thick. 14.6 oz/square yard.

SheerWeave® Series SW2703 by Phifer®: Duplex basketweave fabric—light exterior color combined with dark interior color for thermal comfort and view-through. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80, CAN/ULC S-102. Bacterial and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacterial Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. 3 percent open. .028 inches thick. 14.0 oz/square yard.

SheerWeave® Series SW2705 by Phifer®: Duplex basketweave fabric—light exterior color combined with dark interior color for thermal comfort and view-through. GREENGUARD Gold®. Manufacturer to supply GREENGUARD Gold certificate. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacterial and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacterial Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental Certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 5 percent open. .025 inches thick. 11.8 oz/square yard.

SheerWeave® Series SW2710 by Phifer®: Duplex basketweave fabric—light exterior color combined with dark interior color for thermal comfort and view-through. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacterial and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacterial Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental Certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 10 percent open. .023 inches thick. 10.4 oz/square yard.

SheerWeave® Series SW2900 (2390) by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 101 (Class A Rating), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80, NFPA 701 TM#1 (small scale), IBC Section 803.1.1 (Class A Rating), NFPA 701 TM#2 (large scale). Bacterial and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacterial Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 5 percent open. .017 inches thick. 11.8 oz/square yard.

E Screen™ 1% by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 1 percent open, .020 inches thick, 13.3 oz/square yard.

E Screen™ 3% by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 3 percent open, .017 inches thick, 11.6 oz/square yard.

E Screen™ 5% by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 5 percent open, .016 inches thick, 10.7 oz/square yard.

E Screen™ 10% by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. .016 inches thick. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 10 percent open, .020 inches thick, 10.3 oz/square yd.

M Screen™ 1% by Mermet®: PVC coated fiberglass in 1 by 2 weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 1 percent open, .022 inches thick, 13.12 oz/square yard.

M Screen™ 3% by Mermet®: PVC coated fiberglass in 1 by 2 weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 3 percent open, .022 inches thick, 12.27 oz/square yard.

M Screen™ 5% by Mermet®: PVC coated fiberglass in 1 by 2 weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 5 percent open, .022 inches thick, 11.3 oz/square yard.

9803 3% by Mermet®: PVC coated fiberglass woven mesh. Dark on one side, light on the other. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to UL GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 3 percent open, .025 inches thick, 11.6 oz/square yard.

Soltis Harmony 88 by Ferrari. PVC-coated polyester with pre-tensioning process for dimensional stability. Flame Retardancy: B-s2,d0 / EN 13501-1, M1/NFP92-507, B1/DIN 4102-1, BS 7837, BS 5867, Schwerbrennbar Q1-Tr1/ONORM, A3800-1, CAN/ULC-S 109, M1/UNE 23727-90, Classe 1/EN 13773, CSFM T19, Method 1 & 2/ NFPA 701, Class A/ASTM E84, 1530.3/AS NZS, Group 1/AS NZS 3837, G1/GOST.30244.94. Environment: Certified to UL GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Recyclable through TexyLoop®. 8 percent open, .018 inches thick, 10.61 oz/square yard.

Soltis Horizon 86 by Ferrari. PVC-coated polyester with pre-tensioning process for dimensional stability. Flame Retardancy: B-s2,d0 / EN 13501-1, M1/NFP92-507, B1/DIN 4102-1, BS 7837, BS 5867, Schwerbrennbar Q1-Tr1/ONORM, A3800-1, CAN/ULC-S 109, M1/UNE 23727-90, Classe 1/EN 13773, CSFM T19, Method 1 & 2/ NFPA 701, Class A/ASTM E84, 1530.3/AS NZS, Group 1/AS NZS 3837, G1/GOST.30244.94. Environment: Certified to UL GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Recyclable through TexyLoop®. 14 percent open, .018 inches thick, 11.2 oz/square yard.

Soltis® Perform 92 4% by Serge Ferrari®: PVC-coated polyester with silver backing on outside and pre-tensioning process for dimensional stability. Fire rating: Method 1/NFPA 701, CSFM T19, Class A/ ASTM E84. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Recyclable through TexyLoop. 4 percent open, .018 inches thick, 12.4 oz/square yard.

Color and pattern: [<insert fabric color>, No. <insert fabric number>][As selected by Architect from manufacturer’s full range of standard colors and patterns].

Custom Printed Graphics: As submitted to manufacturer in accordance with manufacturer’s guidelines.

Retain the Window Size paragraph below if window size is known and uniform throughout the project. If there are different window sizes in the project, copy and paste this paragraph once for each different size; designate a different window opening type for each different size; and provide the dimension for each window type.

If window sizes are unknown or there is a large number of different size windows, delete the paragraph below and be sure the window sizes are clearly indicated on the Drawings.

Window <Insert window designation used on Drawings> Size: <insert dimension> inches wide by <insert dimension> inches tall.

CONTROLS

Select one of the following motor option paragraphs and delete the ones not required. Contact Draper for more information concerning motor limitations.

Controls for standard Radio Technology Motor-110 VAC motor with built-in radio receiver.

Select the control device(s) required and delete those not required.

Individual Control, Group Control and Individual and Group Control:

Single channel wireless handheld transmitter-Patio Design.

Four channel wireless handheld transmitter- Patio Design.

Dry Contact - Allows for full control of radio motor shade via dry contact interface.

Wind Sensor- Allows operation of one group of radio motors per timer-Patio Remote Required.

Universal RTS Interface (16-channel)- system integration.

Controls for 110 VAC wired motor.

Select one of the following control option paragraphs and delete the ones not required. For each control selected select the control device(s) required and delete those not required.

Controls:

1MC AC Motor Controller: One per motor. Independently operated with configurable responses to wall switches, RF remotes, and signals from third-party systems.

4MC Four AC Motor Controller: Controls up to four motors. Each port independently operated with configurable responses to wall switches, RF remotes, and signals from third-party systems.

ISO relay - ETL listed component. One ISO relay per motor. Allows 110-120V group switching via toggle switch. Allows for up to 12 motors on one switch.

Network Devices:

Network Device Connector: Connects devices into a network. Distributes power and communications between devices. One provided with each network device. LED indicators for network status. Includes built-in terminating resistor, and network bypass switch for troubleshooting.

Wireless Network Gateway. Allows wireless devices to control shades on the wired network. Required when using Radio Frequency handheld remote. Approved for use in plenum spaces. LEDs indicate the status of both the wired and wireless networks. Allows wireless control of the entire network from a single receiving point. Material: ABS plastic. Operating temperature: Ambient. Dimensions: 3 3/8" x 2" x 1 1/16" (85 x 52 x 27mm). Mounting hardware provided. Indoor use only.

A\V Gateway. Used to integrate with third party control systems. Single entry point must support RS-232 and RS-485 and contain at least 6 contact closure inputs, low voltage trigger, and IR receiver input. Approved for use in plenum spaces. LCD display allows for commissioning of serial groups. Material: ABS plastic. Operating temperature: Ambient. Dimensions: 4 7/8" x 3" x 1" (124 x 83 x 32m). Mounting hardware provided. Indoor use only.

Contact Closure Interface. Allows up to 6 contact closure inputs to be connected to the network. These are grouped into 3 sets of Up/Down signals, each of which can control any number of shades on an lntelliFlex I/O network. Configuration must be done using onboard programming buttons, without rewiring or the use of external configuration devices. Power is provided through the lntelliFlex I/O network using a single Ethernet cable for power and communication. Material: ABS plastic. Operating temperature: Ambient. Dimensions: 3" x 3" x 1" (76 x 76 x 25mm). Mounting hardware provided. Indoor use only.

Sensor/Schedule Interface. Allows up to 4 sensors to be added to an IntelliFlex I/O network. Must include built-in support for brightness, temperature, wind, and precipitation sensors, and pyranometers. Must support up to 8 scheduled override events. Configuration must be done using on-board buttons and LCD display. Supports 0-10V, 4-20mA, photodiode and pulse sensors. External power input for sensors provides support for any sensor power requirements. Material: ABS plastic. Operating temperature: Ambient. Dimensions: 4 7/8” x 3¼” x 1¼” (124 x 83 x 32m). Mounting hardware provided. Indoor use only.

SSI Transformer. To be used when adding third party sensors to the network.

Sensor/Schedule Interface with Automated Glare Control. The IntelliFlex® I/O Sensor / Scheduler Interface with automated glare control allows up to 4 sensors to be added to an IntelliFlex I/O network. Includes built-in support for brightness, temperature, wind, and precipitation sensors as well as pyranometers. Also provided support for up to 8 scheduled override events. Configuration is done using on-board buttons and LCD display. Supports 0-10V, 4-20mA, photodiode and pulse sensors. Configuration using built-in buttons and LCD display. Scheduled events can be configured to run on any combination of days of the week. External power input for sensors provides support for any sensor power requirements. Material: ABS plastic. Operating temperature: Ambient. Dimensions: 4 7/8" x 3" x 1" (124 x 83 x 32m). Mounting hardware provided.

1 Motor Controller. Single motor controller for use with motorized roller shades, Venetian blinds, and louver systems. Can be used as a standalone device or part of a network. When networked, can still be independently operated and can be configured to respond to wall switches, RF remotes, and signals from 3rd party systems. On-board contact closure interface. RS-485 network with built-in network termination. On-board buttons for testing and configuring motors. Pluggable terminal blocks. Input power: 110-230VAC @ 50 / 60Hz. Output power: Maximum 6A at input power. Material: ABS plastic. Operating temperature: 32 - 140 ° F (0 - 60° C). Dimensions: 2¾” x 3½” x 2 1/8” (70 x 90 x 54mm). Enclosure: 4" x 4" X 4" (102 x 102 x 102mm).

4 Motor Controller. 4 motor controller for use with motorized roller shades, Venetian blinds and louver systems. Each port is independently operated and can be configured to respond to wall switches, RF remotes, and signals from 3rd party systems. RS-485 network with built-in network termination. Pluggable terminal blocks. Input power: 110-230VAC @ 50 / 60Hz. Output power: Maximum 6A at input power. Material: ABS plastic. Operating temperature: 32 - 140 ° F (0 - 60° C). Dimensions: 3½” x 7” x 1 7/8” (89 x 178 x 48mm). Enclosure: 6" x 4" x 4" (152 x 102 x 102mm).

Central Network Controller. Touchscreen device to add advanced functionality to a network. It also acts as a router between the control network and a building IP network. Graphical user interfaces on local touchscreen as and remote access web interfaces. Automated glare control and scheduling. BACnet/IP integration. Interface to monitor and override network devices. Allows logging of shade movements, sensor readings, and user overrides. Input power: Micro USB. Operating temperature: ambient. Dimensions: 9 7/8" x 5¾" x 2" (251 x 146 x 51mm). Rough cutout: 8" x 5" x 1¾" (203 x 127 x 44mm). Mounting hardware provided.

IP Gateway. For connecting a network of IntelliFlex I/O devices to an IP network. Built-in two port network switch for connecting directly to a building network. Can be daisy chained to other IPGs to form a stand-alone network for access to all connected IntelliFlex I/O devices. Each IPG is a BACnet/IP device and allows for an additional 120 devices to be added to the network. Configuration is done using the built-in buttons and display. Material: ABS plastic. Operating temperature: ambient. Dimensions: 4 7/8" x 3" x 1" (124 x 83 x 32mm). Mounting hardware provided.

** NOTE TO SPECIFIER ** Select required input device option paragraphs and delete the ones not required.

IntelliFlex I/O User Input Devices.

Single Zone Wall Switch. Allows user to move roller shades to any position with a single touch. Swipe gestures must be recognized to move shades to fully open or closed positions. Power provided through lntelliFlex I/O network using a single Ethernet cable for power and communication. Switch shall be capable of controlling any number of shades on an lntelliFlex I/O network using onboard programming buttons without rewiring or the use of external configuration devices. Must allow unlimited number of preset alignment positions. Push button switches not permitted.

Dual Zone Wall Switch. Allows user to control two independent zones of roller shades, moving them to any position with a single touch. Swipe gestures must also be recognized to move shades to fully open or closed positions. Power for switch must be provided through the lntelliFlex I/O network using a single Ethernet cable for power and communication. Each zone must be configured to control any number of shades on an lntelliFlex I/O network using the onboard programming buttons, without rewiring or the use of external configuration devices. Unlimited number of preset alignment positions. Push button switches not permitted.

** NOTE TO SPECIFIER ** IntelliFlex I/O Wireless Gateway is required with this device.

Radio Frequency Handheld Remote Control. Display must allow for any number of channels, up to 32 channels. Touch control to position the shades anywhere with one touch. Must be capable of serving as a simple commissioning tool. lntelliFlex I/O Wireless Network Gateway is required with remote. Range is 40 feet from closest IntelliFlex I/O device. Push button remotes not permitted.

** NOTE TO SPECIFIER ** IntelliFlex I/O A\V Gateway is required with this device.

Infrared (IR) Remote Control Kit. Includes handheld remote transmitter and IR receiver. For use with A\V Gateway.

EXECUTION

EXAMINATION

Do not begin installation until substrates have been properly prepared.

If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

PREPARATION

Coordinate requirements for blocking and structural supports to ensure adequate means for installation of outdoor motorized tension zip shades.

Coordinate requirements for power supply conduit, and wiring required for outdoor motorized tension zip shades and controls.

INSTALLATION

Install in accordance with manufacturer's instructions.

Install outdoor motorized tension zip shades level, plumb, square, and true. Allow proper clearances for window operation hardware.

Position shades level, plumb, and at proper height relative to adjacent construction. Secure with fasteners recommended by manufacturer.

Include the following paragraph for if systems are to be used for high levels of light exclusion. Delete if not applicable.

Install headbox, side channels, and sill channel with sealant specified in Section 079200 “Joint Sealants” to eliminate light leaks at perimeter of shade system.

TESTING AND DEMONSTRATION

Test outdoor motorized tension zip shades to verify that operating mechanism, fabric retainer, and other operating components are functional. Correct deficiencies.

Demonstrate operation of shades to Owner's designated representatives.

PROTECTION

Protect installed products until completion of project.

Touch-up, repair or replace damaged products before Substantial Completion.

SCHEDULES

Retain Paragraph below to suit project requirements. Identify products by name on the Drawings or alternately use this paragraph to define the location of each type of material to be used. Delete if not required.

Refer to Drawings for exterior motorized window zip shade types and locations.

END OF SECTION