SECTION 11 52 13

PARAGON E FRONT PROJECTION SCREENS

Display hidden notes to Specifier.

** NOTE TO SPECIFIER **  Draper Inc; Wall and ceiling surface mounted and recess mounted front projection screens.

This section is based on the products of Draper, Inc., whose sales office and manufacturing facility are located at:

411 S. Pearl,  P. O. Box 425

Spiceland, IN 47385-0425

Toll Free Tel: 800-238-7999

Tel: 765-987-7999

Email: drapercontract@draperinc.com

Web: www.draperinc.com

Draper manufactures the best and most complete line of projection screens in the world. We want to help you incorporate these screens into the most effective presentation systems. Planning a projection system involves several steps: choosing the screen size, viewing surface, screen model and control system if required. For additional information, go to www.draperinc.com.

GENERAL

SECTION INCLUDES

** NOTE TO SPECIFIER **  Delete items below not required for project.

Electrically operated, ceiling recessed, front projection screens.

Front projection screen controls.

RELATED SECTIONS

** NOTE TO SPECIFIER **  Delete any sections below not relevant to this project; add others as required.

Division 5 - Metal Fabrications: Suspension systems for projection screens.

Section 06 40 00 - Interior Architectural Woodwork: Wood trim for recessed screen installation.

Section 09 22 26 - Ceiling Suspension System: Supports and trim for suspended ceilings.

Section 09 26 13 - Gypsum Plaster: Ceiling for recessed screen installation.

Section 09 21 16 - Gypsum Board Assemblies: Ceiling for recessed screen installation.

Section 09 51 23 - Acoustical Tile Ceilings: Ceiling for recessed screen installation.

Division 26 for electrical wiring, connections, and installation of remote control switches for electrically operated projection screens.

REFERENCES

** NOTE TO SPECIFIER **  Delete references from the list below that are not actually required by the text of the edited section.

NFPA 70 - National Electrical Code.

NFPA 701-99 - Fire Tests for Flame-Resistant Textiles and Films.

GREENGUARD Environmental Institute Gold.

US Green Building Council.

SUBMITTALS

Submit under provisions of Section 01300.

Product Data:  Manufacturer's data sheets on each product to be used, including:

Preparation instructions and recommendations.

Storage and handling requirements and recommendations.

Installation methods.

Wiring diagram for electrically operated units.

** NOTE TO SPECIFIER **  Retain below for front projection screens where shop drawings are needed to understand relationships with adjoining work.

Shop Drawings: Shop drawings showing layout and types of projection screens. Show the following:

** NOTE TO SPECIFIER **  Edit below to suit screens specified and project conditions.

Location of screen centerline.

Location of wiring connections.

Seams in viewing surfaces.

Detailed drawings for concealed mounting.

Connections to suspension systems.

Anchorage details.

Accessories.

** NOTE TO SPECIFIER **  Delete selection samples if colors have already been selected.

Selection Samples:  For each finish product specified, two complete sets of color chips representing manufacturer's full range of available colors and patterns.

Verification Samples:  For each finish product specified, two samples, minimum size 6 inches (150 mm) square, representing actual product, color, and patterns.

QUALITY ASSURANCE

Single Source Responsibility:  Obtain each type of projection screen required from a single manufacturer as a complete unit, including necessary mounting hardware and accessories.

Coordination of Work:  Coordinate layout and installation of projection screens with other construction supported by, or penetrating through, ceilings, including light fixtures, HVAC equipment, fire-suppression system, and partitions.

DELIVERY, STORAGE, AND HANDLING

Do not deliver projection screens until building is enclosed and other construction where screens will be installed is substantially complete.

Store products in manufacturer's unopened packaging until ready for installation.

Protect screens from damage during delivery, handling, storage, and installation.

COORDINATION

Coordinate work with installation of ceilings, walls, electric service power characteristics, and location.

WARRANTY

Manufacturer limited warranty: 5 years from date of purchase.

PRODUCTS

MANUFACTURERS

Acceptable Manufacturer: Draper, Inc. Sales offices and manufacturing facility located at: 411 S. Pearl P. O. Box 425; Spiceland, IN  47385-0425.  ASD.  Toll Free Tel: 800-238-7999; Tel: 765-987-7999; Web: www.draperinc.com.

** NOTE TO SPECIFIER **  Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

Substitutions:  Not permitted.

Requests for substitutions will be considered in accordance with provisions of Section 01 60 00.

MOTORIZED, CEILING RECESSED, FRONT PROJECTION SCREENS

Paragon E: Large electrically operated, extruded aluminum case. Projection screen with motor in roller. 13-1/4 inches high x 12 inches deep (337 mm high x 305 mm deep) case fully enclosed except for slot allowing viewing surface passage. Roller: 6 inches (152 mm) diameter steel tube. Viewing surface securely attached to roller at top and at bottom to weighted dowel. Provided with universal mounting brackets for wall, ceiling or above ceiling mounting. Viewing surface is concealed when not in use; lower it to any position at the touch of a switch.

Motor UL certified, rated 110-120V AC, 60 Hz, three wire, instantly reversible, lifetime lubricated with pre-set accessible limit switches.

** NOTE TO SPECIFIER **  If wall mounting this screen, optional wall mounting brackets are available. Delete the following paragraph if wall mounting brackets are not required.

Wall Mounting Bracket Kit: Kit includes two structural steel brackets and hardware to attach brackets to case. Hardware for mounting to wall structure by others

Projection Viewing Surface:

Matt White XT1000E - On Axis gain of 1.0. 180 degree viewing cone. Washable surface. GREENGUARD Gold certified. 4K ready.

** NOTE TO SPECIFIER ** Select the screen format and size required for the project. Delete the paragraphs not required.

Viewing Area H x W.

HDTV Format (16:9). Black masking borders standard.

270 inch (6858 mm) diagonal, 133 inches x 236 inches (3378 mm x 5994 mm).

300 inch (7620 mm) diagonal, 146 inches x 260 inches (3708 mm x 6604 mm)

324 inch (8230 mm) diagonal, 160 inches x 284 inches (4216 mm x 7214 mm).

354 inch (8992 mm) diagonal, 173 inches x 308 inches (4392 mm x 7823 mm).

384 inch (9754 mm) diagonal, 187 inches x 332 inches (4750 mm x 8433 mm).

16:10 Format. Black masking borders standard.

278 inch (7061mm) diagonal, 147-1/2 inches x 236 inches (3747 mm x 5994 mm).

307 inch (7798 mm) diagonal, 162-1/2 inches x 260 inches (4128 mm x 6604 mm)

335 inch (2159 mm) diagonal, 177-1/2 inches x 284 inches (4508 mm x 7214 mm).

363 inch (2769 mm) diagonal, 192-1/2 inches x 308 inches (4890 mm x 7823 mm).

392 inch (3480) diagonal, 207-1/2 inches x 332 inches (5270 mm by 8433 mm).

** NOTE TO SPECIFIER ** If an extra screen drop is required for the project, fill in the drop height and select the screen border type from one of the following paragraphs and delete the other. Total screen height cannot exceed 24 feet (7.31 m). If not required, delete both paragraphs.

Provide an extra screen drop with an overall screen drop of ___ inches (___ mm) with top border matching the viewing surface.

Provide an extra screen drop with an overall screen drop of ___ inches (___ mm) with a black masking top border.

** NOTE TO SPECIFIER ** Select controls and wall and/or remote-control switches required for project. Delete the types of controls and switches not used on the project. Coordinate the compatibility of multiple control selections.

FRONT PROJECTION SCREEN CONTROLS

General: All controls are UL Certified.

** NOTE TO SPECIFIER ** Not compatible with options 2, 3, 4, 7, 8, 10, 11 or 12

Single station control rated 115V AC, 60 Hz with 3-position rocker switch with cover plate to stop or reverse screen at any point.

** NOTE TO SPECIFIER ** Not compatible with options 1, 6, 7, 10, 11 or 12.

Low voltage control unit with three button 24V switches and cover plate to stop or reverse screen at any point, built-in RF receiver, built-in Video Interface Control trigger for 3V-28V, RS232, and dry contact relays.

** NOTE TO SPECIFIER ** . Not compatible with options 1, 6, 7, 10 or 12.

Low voltage 24V control unit with hand held RF remote three button control switch to stop or reverse screen at any point, built-in RF receiver, built-in Video Interface Control trigger for 3V-28V, RS232, and dry contact relays.

** NOTE TO SPECIFIER ** Not compatible with options 1, 6, 7, 10, 11 or 12.

Low voltage 24V control unit with hand held IR remote three button control switch to stop or reverse screen at any point, built-in RF receiver, built-in Video Interface Control trigger for 3V-28V, RS232, and dry contact relays.

** NOTE TO SPECIFIER ** Not compatible with options 6, 7, 8, 10, 11 or 12.

Key Operated power supply switch to control power to control system.

** NOTE TO SPECIFIER ** Not compatible with options 3, 4, 5, 6, 7, 10, 11 or 12.

Locking switch cover plate for limited access to three position switch.

** NOTE TO SPECIFIER ** . Not compatible with options 1, 2, 3, 4, 5, 6, 8, 10, 11 or 12.

Key operated 3-position control switch rated 115V AC, 60 Hz to stop or reverse screen at any point.

** NOTE TO SPECIFIER ** Not compatible with options 1, 5, 6, 7, 10, 11 or 12.

3-position low voltage control switch with key locking cover plate rated 24V to stop or reverse screen at any point.

** NOTE TO SPECIFIER ** LVC-IV Required. Not compatible with options 4, 5, 7, or 8.

LVC-IP Bridge. Acts as an IP to Serial Gateway for controlling Draper lifts & screens when used in conjunction with an LVC-IV. Configuration is done using built-in buttons and display.

** NOTE TO SPECIFIER ** Left hand motor location is standard; right hand is optional. Select required paragraph and delete paragraph not required. Not all screens can have the motor mounted on the right end of the case. Consult Draper literature for details.

Motor shall be left mounted (standard).

Motor shall be right mounted.

EXECUTION

EXAMINATION

Do not begin installation until substrates have been properly prepared.

Verify rough-in openings are properly prepared.

If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

PREPARATION

Clean surfaces thoroughly prior to installation.

Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

INSTALLATION

Install in accordance with manufacturer's instructions.

Install front projection screens with screen cases in position and relationship to adjoining construction as indicated, securely anchored to supporting substrate, and in manner that produces a smoothly operating screen with plumb and straight vertical edges and plumb and flat viewing surfaces when screen is lowered.

Test electrically operated units to verify that screen, controls, limit switches, closure and other operating components are in optimum functioning condition.

PROTECTION

Protect installed products until completion of project.

Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION